### Chapter 10: Error Detection and Correction

This chapter explains how errors occur in data communication systems and discusses various methods to detect and correct them. The focus is on ensuring reliable data transfer in the presence of noise or interference.

#### 10.1 Types of Errors

- Single-Bit Error:

  - Only one bit is altered.

  - Common in parallel transmission.

- Burst Error:

  - Two or more bits are changed.

  - Common in serial transmission and often caused by impulse noise.

#### 10.2 Redundancy

- Definition: Adding extra bits to the data to detect or correct errors.

- Common techniques include:

  - Error Detection: To find errors.

  - Error Correction: To locate and correct errors.

#### 10.3 Detection Methods

- Parity Check:

  - Adds a single parity bit to the data.

  - Even Parity: Ensures the total number of 1s is even.

  - Odd Parity: Ensures the total number of 1s is odd.

- Cyclic Redundancy Check (CRC):

  - Based on binary division.

  - Sender appends a CRC code to the data; receiver recalculates CRC to check for errors.

- Checksum:

  - Data is divided into fixed-sized segments.

  - The sum of the segments is sent as a checksum.

#### 10.4 Correction Methods

- Hamming Code:

  - Adds redundancy bits for error correction.

  - Can detect up to 2-bit errors and correct single-bit errors.

- Block Coding:

  - Encodes data in blocks, adding redundancy for error detection and correction.

- Convolutional Coding:

  - Processes input bits continuously with memory.

  - Widely used in wireless and satellite communications.

#### 10.5 Forward Error Correction (FEC)

- The receiver corrects errors without retransmission.

- Common in real-time systems where retransmission isn't feasible.

#### 10.6 Automatic Repeat Request (ARQ)

- The sender retransmits data if errors are detected.

- Three main types:

  - Stop-and-Wait ARQ

  - Go-Back-N ARQ

  - Selective Repeat ARQ

#### 10.7 Summary

- Error detection and correction are vital for reliable communication.

- The choice of technique depends on system requirements like latency, bandwidth, and error tolerance.

### Cyclic Redundancy Check (CRC)

The Cyclic Redundancy Check (CRC) is an error-detection method widely used in digital networks and storage devices to ensure data integrity. It is based on binary division and provides high accuracy in detecting errors. CRC appends a sequence of check bits, known as the CRC code, to the original data.

### How CRC Works

#### 1. Key Components

- Dataword: The original data to be transmitted.

- Generator Polynomial (G(x)): A predefined binary polynomial used as a divisor.

- Codeword: The combination of the dataword and the CRC.

### Checksum

The checksum is an error-detection method widely used in network protocols and data transmission to ensure data integrity. It works by summing up data values and comparing the computed checksum at the sender and receiver. If there is a mismatch, it indicates that an error occurred during transmission.

### How Checksum Works

#### 1. Key Concept

- The checksum is the binary sum of data blocks.

- Both the sender and receiver compute this sum and verify its correctness.

#### 2. Steps of Checksum Calculation

##### At the Sender:

- Divide Data into Equal-Sized Blocks:

  - The data is divided into blocks of fixed size (e.g., 16 bits).

- Compute the Sum:

  - Perform a binary addition of all data blocks. If the sum exceeds the block size, the overflow bits (carry) are added back to the least significant bits.

- Complement the Sum:

  - Take the one's complement (invert all bits) of the sum. This value is the checksum.

- Transmit Data and Checksum:

  - The sender appends the checksum to the data and sends it to the receiver.

##### At the Receiver:

- Divide Data and Checksum:

  - The receiver divides the received data (including the checksum) into equal-sized blocks.

- Compute the Sum:

  - Perform binary addition of all blocks, including the checksum. Add overflow bits back to the least significant bits.

- Check the Result:

  - If the final sum is all 1s (in binary), the data is error-free. Otherwise, an error is detected.

### Advantages

- Efficient single-bit error correction.

- Simple to implement in hardware or software.

- Widely used in memory systems (e.g., ECC RAM).

### Limitations

- Can detect but not correct two-bit errors.

- Overhead increases with data size due to added redundant bits.

Chapter 11, "Data Link Control (DLC)

focuses on the mechanisms, techniques, and protocols used in the data link layer of the OSI model. This chapter explains how the data link layer ensures reliable communication between two directly connected nodes in a network. Here’s a breakdown of the main topics typically covered in this chapter:

### 1. Data Link Layer Responsibilities

- Framing: The process of dividing the stream of bits from the network layer into manageable data units (frames).

- Flow Control: Ensures that a sender does not overwhelm the receiver with too much data at once.

- Error Control: Detects and corrects errors during data transmission.

- Access Control: Coordinates access to the shared physical medium when multiple devices are present.

### 2. Framing

Framing defines how data is packaged for transmission. It includes:

- Character-oriented framing: Uses special characters to mark frame boundaries.

- Bit-oriented framing: Uses bit patterns (like flags) to indicate frame boundaries.

- Error detection bits: Added as part of the frame to detect errors.

### 3. Flow Control Mechanisms

These mechanisms regulate data flow to avoid buffer overflow. Key techniques include:

- Stop-and-Wait Protocol: The sender transmits a frame and waits for an acknowledgment before sending the next frame.

- Sliding Window Protocol: Allows multiple frames to be sent before requiring an acknowledgment, improving efficiency.

### 4. Error Control

Ensures reliable delivery through:

- Error Detection: Identifies errors in received frames using techniques like checksums or CRC (Cyclic Redundancy Check).

- Error Correction: Uses techniques like ARQ (Automatic Repeat Request) to retransmit corrupted frames. ARQ methods include:

  - Stop-and-Wait ARQ

  - Go-Back-N ARQ

  - Selective Repeat ARQ

### 5. Data Link Layer Protocols

- Simple Protocol: A basic protocol with no flow or error control.

- Stop-and-Wait Protocol: Implements flow control but has limited efficiency.

- Sliding Window Protocol: Offers both flow and error control with better throughput.

### 6. Access Control

- When multiple devices share a medium, access control mechanisms manage who can send data at a given time.

- Polling and Token Passing are commonly used methods.

### 7. Examples of DLC Protocols

- HDLC (High-Level Data Link Control): A widely-used bit-oriented protocol for reliable communication.

- PPP (Point-to-Point Protocol): Used in point-to-point links like between a computer and ISP.

### Summary

The data link control mechanisms are vital for ensuring reliable and efficient communication. This chapter provides theoretical and practical insights into how protocols manage issues such as flow control, error control, and medium access. Key techniques like Stop-and-Wait ARQ and Sliding Window Protocols form the foundation for advanced networking systems.

Let me know if you'd like me to elaborate on any specific topic!

### 11.4 POINT-TO-POINT PROTOCOL (PPP)

### One of the most common protocols for point-to-point access is the Point-to-Point Protocol (PPP).

### 1. Services Provided by PPP

#### a. Frame Format Definition

#### b. Link Establishment and Negotiation

#### c. Multi-Network Layer Support

#### d. Optional Authentication

#### e. Multilink PPP (MLPPP)

#### f. Network Address Configuration

### 2. Services Not Provided by PPP

#### a. Lack of Flow Control

#### b. Basic Error Control

#### c. No Sequence Numbering

#### d. Limited Addressing

### Explanation of Authentication Protocols in PPP

Authentication in PPP is a critical phase that ensures only authorized users can access resources. Since PPP is commonly used over unsecured links like dial-up connections, verifying user identity is essential. PPP includes two authentication protocols, PAP (Password Authentication Protocol) and CHAP (Challenge Handshake Authentication Protocol), which are used during the authentication phase.

### 1. Password Authentication Protocol (PAP)

#### Overview

- PAP is a simplistic two-step authentication protocol.

- It requires the user to send a username and password to the system, which verifies their validity.

#### Steps in PAP Authentication

- The user sends an authentication identification (username) and a password to the system.

- The system verifies the credentials and responds with:

  - Authenticate-Ack: Grants access if credentials are correct.

  - Authenticate-Nak: Denies access if credentials are incorrect.

#### Key Features

- Protocol Field in PPP Frame: When carrying PAP packets, the protocol field value is 0xC023.

- Packet Types:

  - Authenticate-Request: Contains the username and password sent by the user.

  - Authenticate-Ack: Sent by the system to confirm authentication success.

  - Authenticate-Nak: Sent by the system to indicate authentication failure.

#### Drawbacks

- Security Issue: PAP transmits passwords in plain text over the network, making it vulnerable to eavesdropping or interception by attackers.

- No Dynamic Challenges: The static exchange of credentials makes it less secure compared to CHAP.

### 2. Challenge Handshake Authentication Protocol (CHAP)

#### Overview

- CHAP uses a three-way handshake for authentication, offering greater security than PAP.

- The user’s password is never transmitted directly, reducing the risk of interception.

#### Steps in CHAP Authentication

- The system sends a Challenge Packet to the user, containing a random challenge value.

- The user applies a hashing function (e.g., MD5) on the challenge value and the password, generating a response value. This value is sent back to the system in a Response Packet.

- The system performs the same hashing process on its stored version of the user’s password and compares the result to the received response:

  - If the two results match, access is granted.

  - If they do not match, access is denied.

#### Key Features

- Dynamic Challenges: The challenge value changes with each authentication attempt, increasing security.

- No Password Transmission: The password itself is never sent over the network, making CHAP more secure.

- Reauthentication: The system can periodically reauthenticate the user during the session by sending new challenge values.

#### Advantages Over PAP

- Even if an attacker intercepts the challenge and response values, the password remains secret, as it is not transmitted.

- The dynamic nature of challenge values ensures that replay attacks are much harder to execute.

### Comparison: PAP vs. CHAP

| Feature | PAP | CHAP |
| --- | --- | --- |
| Security Level | Low (password sent in plain text) | High (password not sent directly) |
| Authentication | Simple two-step process | Three-way handshake |
| Password Handling | Password sent directly | Password hashed before transmission |
| Dynamic Challenges | Not supported | Supported |
| Use Case | Basic, insecure environments | Secure, dynamic environments |

### Chapter 12

### 1. 1-Persistent Method

- If the channel is busy:
The station continuously senses the channel until it becomes idle.

- If the channel is idle:
The station immediately transmits the data.

  - Pros: Simple and efficient when the channel is mostly idle.

  - Cons: Can lead to collisions if multiple stations sense the idle channel and transmit at the same time.

### 2. Nonpersistent Method

- If the channel is busy:
The station waits for a random period before sensing the channel again.

- If the channel is idle:
The station transmits the data.

  - Pros: Reduces collisions compared to 1-persistent.

  - Cons: May lead to increased delay because of the random wait time.

3.p-persistent method

The p-persistent method is used if the channel has time slots with a slot duration equal to or greater than the maximum propagation time.

### How it Works

- When the channel is busy:
The station waits and keeps sensing the channel.

- When the channel is idle:

  - The station does not always transmit immediately. Instead:

    - It transmits with a probability p (a number between 0 and 1).

    - It waits for the next time slot with a probability 1−p.

- If the station doesn’t transmit (with probability 1−p):

  - It will check the channel again in the next time slot.

  - The process repeats until the station either transmits or the channel becomes busy again.

### Why Use Probability?

The p-persistent method helps reduce collisions. If many stations are waiting to transmit, not all of them will send data immediately when the channel becomes idle. The probability ppp spreads out their transmissions over time, reducing the chance that two or more stations transmit at the same time.

CSMA/CD

In this method, a station monitors the medium after it sends a frame to see if the transmission was successful. If so, the station is finished. If, however, there is a collision, the frame is sent again.

Collision Handling:

- When a collision is detected:

  - The station stops transmitting immediately.

  - It sends a jam signal to ensure all other stations know a collision occurred.

- Backoff and Retry:

  - After a collision, each station waits for a random period before attempting to retransmit.

  - This random delay is called exponential backoff (the delay increases exponentially after successive collisions).

  - The process repeats until the transmission is successful.

At time t1, station A has executed its persistence procedure and starts sending the bits of its frame. At time t2, station C has not yet sensed the first bit sent by A. Station C executes its persistence procedure and starts sending the bits in its frame, which propagate both to the left and to the right. The collision occurs sometime after time t2. Station C detects a collision at time t3 when it receives the first bit of A’s frame. Station C immediately (or after a short time, but we assume immediately) aborts transmission. Station A detects collision at time t4 when it receives the first bit of C’s frame; it also immediately aborts transmission. Looking at the figure, we see that A transmits for the duration t4 − t1; C transmits for the duration t3 − t2.

CSMA/CA

### (CSMA/CA) was invented for wireless networks. Collisions are avoided through the use of CSMA/CA’s three strategies: the interframe space, the contention window, and acknowledgments,

### 1. Interframe Space (IFS)

- Purpose: To avoid collisions caused by signals from distant stations that might not have reached the transmitting station yet.

- How it Works:

  - When a station senses the channel as idle, it does not transmit immediately.

  - It waits for a short, predefined time called the IFS.

  - This delay ensures that if another station has already started transmitting (but its signal hasn’t reached yet), the collision is avoided.

- Additional Use:

  - IFS can prioritize stations or frame types:

    - Stations with shorter IFS values have higher priority and can transmit earlier.

### 2. Contention Window

- Purpose: To further reduce the chance of collisions by introducing randomness in which stations start transmitting.

- How it Works:

  - The contention window is a time divided into slots.

  - A station ready to transmit chooses a random number of slots as its waiting time after the IFS.

  - The station senses the channel during each slot:

    - If the channel is idle, the timer continues.

    - If the channel becomes busy, the station pauses the timer and waits for the channel to become idle again.

  - The station resumes counting down its remaining slots once the channel is idle again.

  - This gives priority to stations that have waited the longest because their timers are closer to zero.

- Binary Exponential Backoff:

  - If a station fails to transmit after an attempt (due to a busy channel), the contention window size doubles, increasing the range of random slots.

  - This reduces the chance of repeated collisions during high traffic.

### 3. Acknowledgment (ACK)

- Purpose: To ensure reliable delivery of data frames.

- How it Works:

  - After a frame is successfully transmitted, the receiving station sends a positive acknowledgment (ACK) to confirm receipt.

  - If the sender does not receive the ACK within a certain timeout period, it assumes:

    - The frame was lost in a collision, or

    - The frame was corrupted during transmission.

  - The sender retransmits the data frame after the timeout.

Chapter 19

ICMPv4 in Simple Words:

The IPv4 protocol is great for sending data across networks, but it has two major problems:

- The IPv4 has no error-reporting or error-correcting mechanism
If something goes wrong, like a router not knowing where to send the data or the data's time limit running out, IPv4 can’t let the sender know what happened. For example:

  - A router might discard data because it doesn’t have a route to the destination.

  - The final receiver might throw away incomplete data if all parts don’t arrive on time.

- No Query Mechanism:
IPv4 doesn’t have a built-in way to check if another computer or router is online or to gather information for troubleshooting.

To solve these issues, ICMPv4 (Internet Control Message Protocol version 4) was created. It’s like a helper for IPv4, designed to:

- Notify the sender about errors (e.g., unreachable destinations or expired time limits).

- Allow computers and network managers to send queries (e.g., "Are you online?" or "What's your status?").

Even though ICMPv4 is a separate protocol, it works closely with IPv4. ICMP messages are packaged inside IPv4 data packets, and these packets have a special code (protocol = 1) to show they’re carrying ICMP messages.

ICMP Messages:

ICMP (Internet Control Message Protocol) messages are used to handle problems and exchange information in a network. These messages are divided into two main types:

- Error-Reporting Messages:
These messages let the sender know if something went wrong while processing an IP packet. For example:

  - A router might send an error message if it can’t deliver a packet.

  - A destination device might send an error if it gets incomplete or corrupted data.

- Query Messages:
These messages are used to request and share specific information between devices. For example:

  - A computer can ask if another computer or router is online.

  - A router can help redirect a device's messages to the right path.

### ICMP Message Structure:

- Header (8 bytes): Contains essential information about the message.

  - Type Field: Indicates the type of message (e.g., error or query).

  - Code Field: Explains why the message was sent.

  - Checksum Field: Ensures the message isn’t corrupted during transmission.

- Data Section:

  - For Error Messages, it includes details about the packet that caused the error.

  - For Query Messages, data section carries extra information based on the type of query.

ICMP messages are essential for maintaining and troubleshooting a network, helping devices communicate efficiently and handle problems when they arise.

Ping:

The ping program is a tool used to check if a device (like a computer, server, or router) is online and responding. Here's how it works:

- Sending a Request:

  - The source device (your computer) sends an ICMP echo-request message to the destination device.

  - This message asks, "Are you there?"

- Getting a Reply:

  - If the destination device is online, it replies with an ICMP echo-reply message, saying, "Yes, I’m here."

- Tracking Messages:

  - Ping assigns a unique identifier and a sequence number to each message. The sequence number starts at 0 and increases by 1 with each new message.

- Measuring Round-Trip Time (RTT):

  - Ping calculates how long it takes for a message to go to the destination and back.

  - It does this by recording the send time in the message. When the reply comes back, it subtracts the arrival time from the send time to get the RTT

Traceroute or Tracert in Simple Words:

The traceroute (on UNIX) or tracert (on Windows) program is used to track the route that data packets take from a source (your device) to a destination (e.g., a server). It shows the IP addresses of all the routers the packets pass through.

Here’s how it works step-by-step:

### How Traceroute Works:

- Using ICMP Error Messages:

  - Unlike ping, which uses ICMP echo-request and echo-reply messages, traceroute relies on two types of ICMP error-reporting messages:

    - Time-exceeded: Sent by routers when a packet's time-to-live (TTL) expires.

    - Destination-unreachable: Sent by the destination host when it receives a packet with an invalid port.

- Finding the Routers Along the Path:

  - Traceroute sends multiple packets, each with a TTL value starting at 1 and increasing by 1 for each packet.

    - TTL: Limits how many hops (routers) a packet can pass through.

    - What happens when TTL expires? A router discards the packet and sends back a time-exceeded ICMP error message.

- Step-by-Step Discovery:

  - First Packet:

    - TTL = 1, so it reaches the first router.

    - The router discards the packet and sends back a time-exceeded error.

    - Traceroute uses this error to learn the first router’s IP address.

  - Second Packet:

    - TTL = 2, so it reaches the second router.

    - The router sends back another time-exceeded error.

    - Traceroute learns the second router’s IP address.

  - Repeats:

    - This process continues until the packet reaches the destination.

- Reaching the Destination:

  - When the packet finally reaches the destination, it is discarded for a different reason:

    - The port number in the UDP datagram is invalid (deliberately chosen this way).

    - The destination sends back a destination-unreachable message (code 3).

  - This tells traceroute that the final destination is reached.

- Output:

  - Traceroute collects all the IP addresses (and sometimes hostnames) from the routers' responses, showing you the path your packets took.

### Key Points:

- Traceroute works by gradually increasing the TTL to discover each router step-by-step.

- It doesn’t need to know the number of routers (hops) in advance; it figures it out as it goes.

- Typically, it checks up to 30 hops unless it reaches the destination sooner.

- The program also measures the time taken for packets to travel to each router.

In short, traceroute is like a detective following the trail of a data packet, finding every stop along the way until it reaches its destination!

Chapter 20

Distance-Vector Routing

The distance-vector (DV) routing uses to find the best route.

In distance-vector routing, the first thing each node creates is its own least-cost tree with the rudimentary information it has about its immediate neighbors. The incomplete trees are exchanged between immediate neighbors to make the trees more and more complete and to represent the whole internet.

Bellman-Ford equation

The Bellman-Ford equation is foundational in distance-vector routing protocols, allowing the determination of the least-cost path between nodes in a network. It works iteratively to compute the shortest paths based on the costs associated with intermediate nodes.

### Conceptual Steps:

- Initialization:

  - Each node knows the direct link costs to its immediate neighbors.

  - Initial costs to all other nodes are set to infinity (∞\infty∞).

- Iterative Updates:

  - Nodes exchange their distance vectors (a table listing their minimum costs to all destinations).

  - Each node uses the Bellman-Ford equation to update its distance vector:

    - If a shorter path to a destination through a neighbor is found, update the cost.

- Convergence:

  - The algorithm converges when no node can find a shorter path to any destination.

### Distance Vectors:

A distance vector is a data structure that stores the least-cost from a node to all other nodes in the network. Each node shares its distance vector with its neighbors in periodic updates.

### Significance in Routing:

- Dynamic Updates: If a link fails or a new link is added, nodes recalculate their paths using the Bellman-Ford equation.

- Distributed Algorithm: Each node independently computes its own routing table based on information from its neighbors.

### Link-State Routing:

Link-State (LS) Routing calculates the optimal path through a network by assessing the "state" or condition of links (edges) between nodes (routers or systems).

### Key Concepts

- Link-State:

  - Represents the condition or cost of a network link.

  - Links with lower costs are preferred for routing decisions.

  - Links with a cost of infinity are considered broken or non-existent.

- Link-State Database (LSDB):

  - A global, synchronized representation of the network topology.

  - Stored as a two-dimensional array (matrix) where each cell represents the cost of a link between two nodes.

  - Every node has an identical copy of the LSDB, ensuring consistent routing decisions.

### How Nodes Build the LSDB

- Data Collection Using Flooding:

  - Initial Step: Each node sends greeting messages to its immediate neighbors (directly connected nodes).

  - Each greeting gathers:

    - Neighbor's identity.

    - Cost of the link to that neighbor.

  - These two pieces of information are combined into a Link-State Packet (LSP).

- Flooding LSPs Across the Network:

  - Process:

    - A node broadcasts its LSP to all neighbors.

    - When a neighbor receives an LSP:

      - If the LSP is newer (checked using a sequence number) or not seen before, it updates its LSDB and forwards the LSP to its other neighbors.

      - If the LSP is older, it is discarded.

  - This process ensures that all nodes eventually receive the latest LSPs while avoiding redundant retransmissions.

- Stopping Flooding:

  - Flooding stops naturally at nodes with a single interface or when no newer LSPs are available for forwarding.

- Comprehensive LSDB:
After all LSPs have been exchanged, each node can construct a complete LSDB that accurately represents the entire network.

### Benefits of Link-State Routing

- Provides dynamic updates for changes in network topology (e.g., link failures or additions).

- Ensures optimal routing paths by using algorithms like Dijkstra's shortest path algorithm.

- Maintains a consistent view of the network across all nodes.

In the distance-vector routing algorithm, each router tells its neighbors what it knows about the whole internet; in the link-state routing algorithm, each router tells the whole internet what it knows about its neighbors.

Formation of Least-Cost Trees:

To create a least-cost tree for itself, using the shared LSDB, each node needs to run the famous Dijkstra Algorithm. This iterative algorithm uses the following steps:

- The node chooses itself as the root of the tree, creating a tree with a single node, and sets the total cost of each node based on the information in the LSDB.

- The node selects one node, among all nodes not in the tree, which is closest to the root, and adds this to the tree. After this node is added to the tree, the cost of all other nodes not in the tree needs to be updated because the paths may have been changed.

- The node repeats step 2 until all nodes are added to the tree.